# Programming Hello

- Press Reset, verify monitor is displayed.

- Select CMF Flash Programming.

- Install VPP_EN & PROG_EN.

- Select Erase.

- Select Program Cmf Flash.

- UPLOAD file "PB_R_M\Hello_a.s19".

- Remove Vpp_En.

- Remove Prog_En. Press RESET, verify Hello is displayed.

## Reinstall PROG_EN. Press RESET, verify monitor is displayed

# Programming Internal Monitor

- Open terminal 9600 baud.

- Enter “DL” then UPLOAD” file ‘PbProgM.S19’ in ram.

- Enter “GO ” then  “3fa000” the enter.

- On board enable Vpp_En.

- On terminal select ‘E’ to erase flash, verify no errors.

- Select ‘P’ then UPLOAD file ‘Pb_R_M\Pb555M.s19’.                                       Use wait for reply character.

- Disable Vpp_En. Remove power. Remove Bdm.

- Apply power, verify monitor displays on terminal.