Dan Zhang

Team Barbie Jeep

Fall 2008

The Dummy’s Guide to Flash on the PB-0555

This is a guide describing the documented approach to loading flash programs onto the PB-0555 board, which is the platform used in EECS373 for the MPC555. This manufacturer-approved approach uses a built-in monitor program residing on the board for programming. Along with this file, several additional items should be contained within the ZIP file:

- Codewarrior.zip

  - In case the PB-0555 board does not have the Flash Programmer/Monitor installed, you will need to use these files to re-program the board.

- Pb555m_modified.s19

  - The file pb555m.s19 inside the Codewarrior.zip file doesn’t work due to a special character at the end of the file. Use this modified file instead.

- PB-555man.pdf

  - The manual for the PB-0555 board. Contains information on the memory programming space (including the ranges for flash, SRAM, etc) as well as how to use the monitor. Also contains details on the jumpers on the board.

- PB555_SCH_revC.pdf

  - Pin-out schematics for the PB-0555 board.

How to Run the Built-in Monitor Program

The built-in monitor program resides in flash memory. It enables you to easily program the flash memory on the board through the COM port (serial). To program the flash memory, you use the AxIDE program supplied by the Axiom Manufacturing website. You can download AxIDE here: http://www.axman.com/?q=node/171

- Disconnect the board from everything

- Make sure that the PROG_EN jumper is attached. Removing this jumper makes the board load your flash program instead of the monitor.

- Plug in the serial port. Run your terminal application with the following settings:

  - 9800 baud

  - 8 data bits

  - No parity

  - 1 stop bit

  - HARDWARE flow control

  - ANSI emulation

- Plug in the power cable. A green LED should be lit, signifying that the board has power.

- You should see some output on the terminal window. This is the Flash Monitor program.

This list is from the PB-555man.pdf file. If you still have problems, consult the file for solutions. If it still doesn’t work, some earlier group probably erased your flash memory so you’ll need to re-program the monitor.

How to Re-Program the Monitor Using Codewarrior 8.7

So, some asshole group erased the monitor. That’s no problem, thanks to the help that the Axiom Engineers provided for us, we have obtained a guide on programming the Monitor back onto the board. This is contained within the Codewarrior.zip file.

- Extract the files within Codewarrior.zip

- Plug in the BDM cable to the board.

- Run Codewarrior and set the settings as shown in the pictures provided by Codewarrior.zip

- Program the pb555m_modified.s19 file instead of the pb555m.s19 file. The modified version is just the regular version with the last line deleted (contains a special character which the programmer hates).

- Select the shadow memory, as shown in Codewarrior.zip

- Program the shadow memory s19 file. DO NOT ERASE THE FLASH MEMORY BEFORE DOING SO.

- Verify everything with the CRC check.

- Unplug the BDM cable, run the terminal program and plug in the serial port.

- Reset the board – the monitor should now be displayed on the terminal screen.

- If that didn’t work, redo steps 3-9. You probably screwed up somewhere.

Configuring Codewarrior for Programming Flash

Codewarrior must be configured properly, else your generated MOT file won’t be accepted by the board. To accomplish this, both the Codewarrior Flash settings (be sure to not accidentally change the SRAM settings) and the LCF file must be correct. The LCF file must have the following settings:

/* linker command file for 5XX Chip */

_flash_source	= 0x10000;		// **NOTE: MUST match RAM buffer address

// setting in linker preference panel

MEMORY {

ram : org = 0x003f9800

rom : org = 0x00010000 // desired ROM address (boot address for 555)

}

Inside the Flash Settings, set both the RAM buffer address and the ROM address to 0x10000. Now, compile (that’s the button next to the green arrow for compile + run) the code and you will have the MOT file generated correctly (external_FLASH.mot).

Programming Flash Memory

Finally, to program Flash memory, you must use AxIDE as your terminal interface. You cannot have both AxIDE and the windows terminal application open at the same time, since both monitor the serial port.

To program flash memory, follow the section “Programming the CMFI Flash Memory” in the PB-555man.pdf file. First, select option #2 from the Monitor menu choice. Then, erase flash memory. Ignore the message about removing some pin. Then, select the “program” option. Do not hit any key on the keyboard after selecting this option since now all input is interpreted as the hex file to be programmed. Instead, send the s19 or MOT file to the board using AxIDE. Be sure to select the “wait for echo character” option. If you get an error while programming, you most likely didn’t configure Codewarrior correctly.

Comments about Flash Memory

- Code running in Flash memory does so at a different speed than in debug SRAM mode, so you will need to reconfigure your wait loops as a result.

The Next Step

As you have found out, programming flash memory is slow. For debugging purposes, having something fast would be very beneficial. A possible solution is to flash an interrupt handler at the 0x500 location, which redirects to somewhere in SRAM. That way, you can run interrupts in SRAM. However, specifying the address of code in Codewarrior is pretty difficult and we never figured that out.

Good luck on your project!